全息广告机单机协议文件V2.05

Product file

Development document number：：PL20180529

Communication interface instruction

3D led fan

| Name： | taiful |
| --- | --- |
| Valid time： | 2068-05-29 |
| date： | 2018-05-29 |
| version： | V2.05 |

1.System Structure Description

The standalone version is represented by "P" - the device is in hotspot mode, and the control end is connected to the device's WIFI for single device control. The device is in AP mode, and the WIFI name is the brand logo plus "-"+MAC address. For example: XLZ-8CF7106DF9C4， All capitalized, the password is 88888888.

The splicing version uses "S" to represent - the device is connected to a router mode, and the control end is controlled through a local area network connection, without the need for an external network.

The network version is represented by "N" - the device is connected to a router mode, and the control end controls the device through cloud transfer. It requires an external network and can be remotely controlled one to many. The device is connected to the built-in WIFI router named xlz and password xlz123456 by default.

Single machine version connection details:

There are 2 channels for sending control instructions through the WebSocket protocol and uploading videos through HTTP

The device is a server, and upon startup, a WEBSOCKET server connection address of "ws://192.168.43.1:7110" is established.

The control end uploads videos through the HTTP protocol to http://192.168.43.1:8092 ”

If the agreement states "default to 0", just write 0

- 2.Control instruction content

2.0 Common Part - (All JSON part instructions contain this part)

{
	"cmd": 1.//Command word, representing the instruction sent

"source":  0//sender ID integer, default is 0

"destination":  0//Receiver ID integer, default to 0

"order": 1//Send serial number, accumulate once per send, starting from 1 until 255, and then accumulate to 1

"version_code": 1//(1-255), version number, default is 1

"pack_order": 1//(1-255), subcontract number, default is 1

"pack_count": 1//(1-255), number of subcontractors, default is 1

properties:// The different instructions in this section have different data items, which are discussed in the following protocol

{

}

}

2.1 Device to Control End - Single Machine Protocol

2.1.1 Upload device status (device to control end)

"CMD"：0x1A

{

"type":1,//1-Device， Default is 1

"id":  //default to empty

"dev_id":  1//Integer number of the device

"sn":  //The unique code SN of the device

'open': True//When the previous electric rotation state is True Open False Close

"brightness": 15//Brightness value, the larger the brightness (0-15) 0-Darkest 15-Brightest

"packet_type":  0//0- Status packet, 1- Control reply packet, indicating feedback that the control end has sent an instruction, 2- Insufficient device space, 3- Device not online

"file_id": 0//ID of the file being played

WiFi signal strength: 0/(0-100) WiFi signal strength, the larger the signal, the stronger the signal

"model": 0//Backup

"memory": 0//Remaining space of device 0 indicates insufficient space, unit (MB bytes)

"info_show":0//0-no show 1-show

}

example:

{"cmd":26,"destination":0,"order":1,"pack_count":1,"pack_order":1,"properties":{"bluetooth_state":2,"brightness":15,"file_id":0,"id":"6C21A200487B","info_show":0,"ip":"192.168.43.1","memory":3682,"model":0,"open":true,"packet_type":1,"pause":false,"play_mode":1,"type":0,"volume":15,"wifi_signal":0},"source":14903,"version_code":700077}

2.1.2 Feedback on file processing progress (from device to control end)

"CMD"：0x1B//

{

"id": 1//Device id

"dev_id":  1//Integer number of the device

"sn":  //The unique code SN of the device

"percentage_type":  0//Progress type, default is 0

"percentage": 0//(0-100), processing progress

"file_id": 0//Processed file ID

}

2.1.3 Feedback package (from device to control end)

"CMD"：0x1C//

{

"feedback_cmd":  1//Device ID

"state":  0-Failure 1-Success//Device ID

"feedback_index":  1//Feedback Index

}

2.1.4 Report file list (device to control end)

"CMD"：0x1E//

{

"id":  //Device ID

"type_id":  //Feedback type, default is 1

Down_thum ":" true "//true - means the video cover needs to be downloaded through HTTP from the file list in the report, False - means it needs to be downloaded through WebSocket

File:

[

"file_id": 0//File ID

"file_type": 1//1- Image 2- Video

"play_count": 1//The continuous playback frequency of the media

"play_order": The playback order of this media is related to the playback order

"file_name": 1//The file name of the media

"percentage": 100//The download progress of the media, where 100 indicates a complete download

"pack_number": 0//Package number, corresponding to the package number of the video or image data package

"thum_path": The download route for the cover image is: http://192.168.43.1:8092 '+thum_path;

]

}

Example analysis:

After the command is connected to the device and the control terminal, the device actively reports the file list on the device. The control terminal needs to download the cover images of all media in this list and display them. The download link is' http://192.168.43.1:8092 '+thum_path;， The URL is fixed and includes a path

2.1.5 Equipment basic information reporting

"CMD"：0x40//

{

Type ": 1,//0-Device represents the device

"id": ""//Device id

"device_type":2,//0-Device type

"name":  //The SN code of the device, which is the MAC address, should be removed from all:

"password":  //Default is 88888888, WIFI password for standalone mode

"versionNumber":  700074 "//Device software version number 6-digit

"appName":  //The default is empty, which refers to the device prefix name displayed on the APP, given by the server,

"brandName":  The default is empty, which refers to the brand name

"time":// The parameters related to timed power on/off are empty without data

[

"starttime":  2020 12 19 17 35 10 332 "//Start time

"endtime":  2020 12 19 17 35 10 332 "//End time

"devsandweek":  0//needs to be converted to binary. If the first digit is 1, it indicates the day of the week to be executed. For example, 65 indicates Monday and Sunday to be executed

]

}

Explain：

{"properties":{"type":0,"id":"8CF710655A22","name":”8CF710655A22”,"password":”88888888”,"device_type":40"appName":"","brandName":"","versionNumber":"700074"},"cmd":0x40,"source":"1234","destination":"0","order":2,"version_code":1,"pack_order":1,"pack_count":1}

This command is only sent once after the connection is made

2.2 Control End to Device Control Protocol - Single Machine Protocol

2.2.1 Device control instructions (control end to device)

"CMD"：0x2e//

{

"type": 1.//Fixed write 1

"id":  0//Device ID

"value": 0//(0-Shutdown, 1-Power On, 2-Restart, 3-Play, 5-Pause, 6-Play Specific Files, 7-Brightness Adjustment, 0xB Volume Adjustment, 0x10 Angle Adjustment, 0x11 Open Angle, 0x12 Close Angle, 0x20 Splicing Position Test, 0x21 Play Mode, 0x22 Set Speed, 0x23 Forward Rotation, 0x24 Reverse Rotation, 0x25 Power On Rotation, 0x26 Power On Non Rotation, 0x27 Start Stop Memory),

"file_id": 0//is valid when "value=6", indicating the specified file ID to be played

"brightness_value": When 'value=7', specify the brightness of the device, with brightness values ranging from 0 to 15

"adjustment_value": 0//is valid when "value=0x10", when "adjustment-value" is 0, it means the angle decreases, and when 1, it means the angle increases

"volume_value": When 'value=0xB', specify the volume of the device Values ranging from 0 to 15

"play_mode": When the playback mode "value=0x21" is set to 0//, specify the playback mode of the device: 1- sequential loop, 2- single loop, 3- random playback

"speed_value":// When 'value=0x22', the set speed is a number of integers between 600-900, such as 600610620.. 900

}

Shutdown instance:

{"cmd":46,"source":0,"destination":0,"order":3,"pack_order":1,"pack_count":1,"properties":{"type":1,"id":"6C21A200487B","value":0,"file_id":0,"brightness_value":1,"adjustment_value":1,"volume_value":15,"play_mode":1,"speed_value":30},"version_code":1}

Play Mode - Random Play Instance

{"cmd":46,"source":0,"destination":0,"order":4,"pack_order":1,"pack_count":1,"properties":{"type":1,"id":"6C21A200487B","value":33,"file_id":0,"brightness_value":1,"adjustment_value":1,"volume_value":15,"play_mode":3,"speed_value":30},"version_code":1}

Angle+Example

{"cmd":46,"source":0,"destination":0,"order":5,"pack_order":1,"pack_count":1,"properties":{"type":1,"id":"6C21A200487B","value":16,"file_id":0,"brightness_value":1,"adjustment_value":1,"volume_value":15,"play_mode":1,"speed_value":30},"version_code":1}

Example of volume adjustment

{"cmd":46,"source":0,"destination":0,"order":6,"pack_order":1,"pack_count":1,"properties":{"type":1,"id":"6C21A200487B","value":11,"file_id":0,"brightness_value":1,"adjustment_value":1,"volume_value":6,"play_mode":1,"speed_value":30},"version_code":1}

2.2.2 Peripheral configuration instructions (sent to the device end)

"CMD"：0x30//

{

"type":0////0-Device

ID ":" "//Device ID Device ID

"value": (0-Disconnect Bluetooth, 1-Connect Bluetooth)

"bluetooth":  //Bluetooth name

"password":  //Bluetooth password, empty when no password is available

}

3.3.4 System Control Instructions (Cloud To Device) - PN

"CMD"：0x34//

{

"type": 0,//0- represents device

"id":  0//Device ID

"control_type": 1.//1- Update immediately 5- Display device information 8- Open status form 9- Close status form 17- Time interval for device status reporting in seconds 18- Time interval for device file list reporting 19- Display device SN code 20- Do not display device SN code 21- Turn on light testing mode 22- Turn off light testing mode 23- Set time 24- Set color tone "second": 0//controllability is valid when the type is 17 or 18, in seconds, and this number must be a companion number of 20

'date_time ':' 2020 12 19 17 35 10 332 '//The control type is valid at 20 and represents the time

"r_tone":0 //0-65536 32768

"g_tone":0 //0-65536 32768

"b_tone":0 //0-65536 32768

}

example:

{"properties":{"type":1,"id":11,"control_type":0,"second":500},"cmd":52,"source":"0","destination":"8CF710655A22","order":2,"version_code":1,"pack_order":1,"pack_count":1}

explain:

r_tone:32768,g_tone:50000,b_tone: 50000//Cool color value

r_tone:52768,g_tone:32768,b_tone: 32768//Warm color value

r_tone:65536,g_tone:65536,b_tone: 65536//Highlight

r_tone:32768,g_tone:32768,b_tone: 32768//True color value

3.3.5 Time Configuration Instruction (Cloud To Device) - PN

"CMD"：0x35//

{

"type": 0,//0- represents device

"id":  0//Device ID

"time":

[

"id": 0//

"starttime":  2020 12 19 17 35 10 332 "//Start time

"endtime":  2020 12 19 17 35 10 332 "//End time

"devid":  0//Device ID

"mergeid":  0//Device ID

"houseid":  0//Device ID

"devsandweek":  0//needs to be converted to binary, where the first digit represents the day of the week for execution. For example, 65 represents Monday and Sunday for execution, because 65=(1000001), where the first digit corresponds to the day of the week. 1 represents Monday

]

}

example:

3.3.6 Setting up WIFI

"CMD"：0x2F//

{

"type":1,//0-Device

"id":  //Device ID

"mode":  0-Change 1-Add 2-Delete

'ishot': true/false - Set the WiFi password for the connected router, true - Set hotspot

"ssid": ""//wifi is ssid

"password": ""//wifi is password

}

Explain：

{"properties":{"type":1,"id":"8CF710655A22","ssid":"fsp","password":"fsp123456"},"cmd":47,"source":"0","destination":"8CF710655A22","order":4,"version_code":1,"pack_order":1,"pack_count":1}

Change the SSID and password of the device, but please ensure that the device is connected to the network

3.4.3 Prepare to transmit video instructions

"CMD"：0x41//

{

"type": 1.//0-Device 1-Splicing 2-Cluster

"id":  //Device ID

"file_id":  0//File ID

"file_type":  0 1- Image 2- Video

"play_count": 1//Play count

"residence_time": 0//dwell time

"display_mode": 0//Only valid for splicing, "1-in cut, 2-out cut, 3-alone"//Display mode

"level":  0//is only valid for splicing, 0 bits - up 1 bit - right 2 bits - down 3 bits - left, corresponding to a bit of 0 indicating display, 1 indicating no image display

"position":  0//is only valid for splicing, 0 bits - up 1 bit - right 2 bits - down 3 bits - left, corresponding to a bit of 0 indicating display, 1 indicating no image display

"file_name":  //File Name

"pack_number":0//

}

After sending this command, you can now transfer pictures or videos,

If it is an image or transmitted through WEBSOCKET, please refer to section 2.3

If it's a video, upload it through HTTP

3.4.4 Deleting Files

"CMD"：0x43//

{

"type": 1.//Fixed 0

"id":  //Device ID

"file_id":  0//file ID, if file ID is 0, it means clearing all media files

}

explain:

When file_id=0 and type=0, it means deleting all point-to-point videos of the device

When file_id=0 and type=1, it means deleting all spliced videos of the device

When file_id=0 and type=2, it means deleting all network version videos of the device

2.3 Image Upload

This instruction is a bytecode instruction, not in JSON format, and is sent after sending the "prepare to transmit video instruction"

Uploading images is done directly through WEBSOCKET. The original bytecode of the image needs to be extracted and transmitted in packets, with a maximum of 32768 bytes per packet,

Cmd： Command, fixed here as 66

PackNumber: The serial number of this package defaults to 0

PackCount is the total number of packages,

PackIndex is the current packet number sent, starting from 1

Length is the number of bytes of Data

The first row of the table below represents one byte with one square, and the second row represents an explanation

3.4.5 Data Transmission Packet

| 0XFA | 0XF0 |  | 66 |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |  | 0XFE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Head |  | cmd |  | packNumber |  | packCount |  |  |  |  | packIndex |  |  |  |  | length |  |  | Data |  |  |  |

2.4 Video Upload

This command is an HTTP upload command, which is sent after sending the "prepare to transmit video command" and uploaded to http://192.168.43.1:8092 Please note that the uploaded video should not exceed a resolution of 4K. It is best to upload a video with a resolution of 1024 * 1024 or an equal proportion